# PostgreSQL 18 HA Deployment How-To

## Purpose

This guide explains how to deploy the five-node PostgreSQL 18 high-availability platform from a Linux Ansible control machine. It is written as a copy-paste runbook for operators who may not be familiar with Ansible.

The deployment creates:

- a PostgreSQL primary and standby managed by pg_auto_failover;

- a separate pg_auto_failover monitor and WAL archive host;

- two PgBouncer, HAProxy, and Keepalived routing hosts;

- a floating PostgreSQL virtual IP (VIP);

- Chrony time synchronization;

- role-based UFW firewall rules;

- Prometheus node, PostgreSQL, and PgBouncer exporters;

- SSH-based WAL archiving with an optional Rubrik hook.

Destructive storage warning

The storage role partitions and formats the configured data disks. Verify all VMware disks and device names before running site.yml. The OS disk /dev/sda is not modified, but /dev/sdb and /dev/sdc are used according to host role.

Database lv_data must be mounted at /pgdata/pgroot. Do not mount it directly at PGDATA. The directories /pgdata/pgroot/data and /pgdata/pgroot/backup must remain ordinary sibling directories on that XFS filesystem. Existing hosts using the legacy mounted-PGDATA layout require an approved offline migration before this deployment is run.

## 1. Know the hosts

### UAT

| Host | IP | Role | Main services |
| --- | --- | --- | --- |
| BHC-QMSSQLU05 | 192.168.129.105 | Initial database primary | PostgreSQL 18, pg_auto_failover |
| BHC-QMSSQLU06 | 192.168.129.106 | Database standby | PostgreSQL 18, pg_auto_failover |
| BHC-QMSSQLU07 | 192.168.129.107 | Monitor and WAL archive | Monitor PostgreSQL, pg_auto_failover, archive |
| BHC-PGBSQLU03 | 192.168.129.108 | Routing node 1 | Keepalived MASTER, HAProxy, PgBouncer |
| BHC-PGBSQLU04 | 192.168.129.109 | Routing node 2 | Keepalived BACKUP, HAProxy, PgBouncer |

UAT uses VIP 192.168.129.110/24, cluster CIDR 192.168.129.0/24, and application CIDR 192.168.24.0/24.

### Production

| Host | IP | Role | Main services |
| --- | --- | --- | --- |
| BHC-QMSSQLP01.bayshore.ca | 192.168.128.134 | Initial database primary | PostgreSQL 18, pg_auto_failover |
| BHC-QMSSQLP02.bayshore.ca | 192.168.128.135 | Database standby | PostgreSQL 18, pg_auto_failover |
| BHC-QMSSQLP03.bayshore.ca | 192.168.128.136 | Monitor and WAL archive | Monitor PostgreSQL, pg_auto_failover, archive |
| BHC-PGBSQLP01.bayshore.ca | 192.168.128.137 | Routing node 1 | Keepalived MASTER, HAProxy, PgBouncer |
| BHC-PGBSQLP02.bayshore.ca | 192.168.128.138 | Routing node 2 | Keepalived BACKUP, HAProxy, PgBouncer |

Production uses VIP/DNS PGBQMSLSP01 (192.168.128.139/24), cluster CIDR 192.168.128.0/24, application CIDR 192.168.4.0/24, and external Prometheus server BHC-PGMSQLP01 (192.168.128.140).

All hosts must already have:

- Ubuntu 24.04 LTS;

- an ansible user;

- the ansible user’s SSH password;

- passwordless sudo through /etc/sudoers.d/ansible;

- working DNS or IP reachability from the control machine.

## 2. Prepare the Ansible control machine

Use Ubuntu 24.04, another supported Linux distribution, or WSL. Do not use native Windows as the Ansible controller.

### 2.1 Install operating-system prerequisites

sudo apt update
sudo apt install -y git python3 python3-pip python3-venv sshpass

### 2.2 Create an isolated Ansible environment

python3 -m venv ~/.venvs/ansible-pg-ha
source ~/.venvs/ansible-pg-ha/bin/activate
python -m pip install --upgrade pip
python -m pip install "ansible-core>=2.17,<2.20" ansible-lint
ansible-playbook --version

Activate this environment again in every new terminal:

source ~/.venvs/ansible-pg-ha/bin/activate

### 2.3 Enter the repository and install collections

Replace /path/to with the actual parent directory:

cd /path/to/ansible-pg-ha
ansible-galaxy collection install -r requirements.yml
ansible-galaxy collection list

requirements.yml installs:

- ansible.posix;

- community.general;

- community.postgresql;

- community.crypto.

### 2.4 Generate the operator SSH key

The current bootstrap role reads ~/.ssh/id_rsa.pub, so create an RSA key with that exact filename:

mkdir -p ~/.ssh
chmod 700 ~/.ssh
test -f ~/.ssh/id_rsa.pub || ssh-keygen -t rsa -b 4096 -f ~/.ssh/id_rsa
chmod 600 ~/.ssh/id_rsa
chmod 644 ~/.ssh/id_rsa.pub

When ssh-keygen asks for a passphrase, using one is recommended. Load it into the SSH agent before running Ansible:

eval "$(ssh-agent -s)"
ssh-add ~/.ssh/id_rsa

## 3. Configure environment values before deployment

### 3.1 Review UAT and Production addresses

Open both environment files:

nano group_vars/uat.yml
nano group_vars/prod.yml

Verify or replace:

- every ansible_host address in the selected inventory file;

- every host IP in host_ips;

- cluster_cidr;

- prometheus_scrape_cidr;

- every approved subnet in application_client_cidrs;

- postgresql_vip and postgresql_vip_prefix;

- logstash_ip;

- ntp_servers.

The repository’s current environment network matrix is:

| Setting | UAT | Production |
| --- | --- | --- |
| Inventory | inventories/uat_hosts.ini | inventories/prod_hosts.ini |
| Cluster CIDR | 192.168.129.0/24 | 192.168.128.0/24 |
| Application CIDR | 192.168.24.0/24 | 192.168.4.0/24 |
| VIP | 192.168.129.110/24 | 192.168.128.139/24 |
| VIP DNS | Not defined | PGBQMSLSP01 |
| Prometheus source | 192.168.129.0/24 | 192.168.128.140/32 |

The ansible_host address and the corresponding host_ips value must match. The former controls Ansible SSH connections; the latter is installed in the managed PostgreSQL HA block in /etc/hosts on all five servers.

The inventory choice controls which environment group variables Ansible loads:

- inventories/uat_hosts.ini creates the uat group;

- inventories/prod_hosts.ini creates the prod group.

### 3.2 Review common settings

nano group_vars/all.yml

Confirm at least:

- environment-specific keepalived_interface matches the VIP-facing routing interface (ens33 in the documented UAT environment);

- PostgreSQL memory values are appropriate for 32 GB database hosts;

- all storage devices and logical-volume sizes match VMware provisioning;

- ufw_ssh_allowed_cidrs contains the operator/control network;

- ufw_database_allowed_cidrs contains only approved database clients;

- NTP, exporter, VIP, and service ports are correct.

Firewall warning

ufw_reset_rules: true resets UFW on every playbook run and rebuilds only the declared role rules. Add approved exceptions to the Ansible role before deployment. Manually added rules will be removed on the next run.

### 3.3 Store secrets with Ansible Vault

Do not leave real passwords in group_vars/uat.yml, group_vars/prod.yml, or group_vars/all.yml.

Generate one inline Vault block at a time. This helper reads the secret without echoing it or placing it in shell history:

vault_var() {
  var_name="$1"
  read -rsp "Value for ${var_name}: " var_value
  echo
  printf '%s' "$var_value" |
    ansible-vault encrypt_string --ask-vault-pass --stdin-name "$var_name"
  unset var_value
}

Run it for every secret:

vault_var pg_auto_failover_monitor_password
vault_var pg_auto_failover_replication_password
vault_var app_db_password
vault_var postgres_exporter_password
vault_var haproxy_health_password
vault_var keepalived_auth_pass

Each command prints YAML similar to:

app_db_password: !vault |
          $ANSIBLE_VAULT;1.1;AES256
          ...

Copy each complete block into group_vars/uat.yml, replacing the corresponding CHANGE_ME line. Generate a separate set of values and put those blocks in group_vars/prod.yml. The Keepalived VRRP PASS value must be no more than eight characters.

Do not create group_vars/uat/ or group_vars/prod/ directories alongside the existing environment files; that changes how Ansible discovers group variables.

Test that the inventory resolves correctly:

ansible-inventory -i inventories/uat_hosts.ini --graph
ansible-inventory -i inventories/uat_hosts.ini --host BHC-QMSSQLU05 \
  --ask-vault-pass

ansible-inventory -i inventories/prod_hosts.ini --graph
ansible-inventory -i inventories/prod_hosts.ini \
  --host BHC-QMSSQLP01.bayshore.ca --ask-vault-pass

## 4. Step 1 — Push the SSH public key

The initial connection uses the existing ansible account password. Run:

ansible-playbook -i inventories/uat_hosts.ini bootstrap.yml --ask-pass --ask-vault-pass

At SSH password:, enter the SSH password for the ansible account. Input is not displayed while typing.

For Production:

ansible-playbook -i inventories/prod_hosts.ini bootstrap.yml --ask-pass --ask-vault-pass

The bootstrap play:

- creates /home/ansible/.ssh with mode 0700;

- installs the control machine’s ~/.ssh/id_rsa.pub;

- enforces authorized_keys mode 0600;

- leaves ownership as ansible:ansible.

### Verify passwordless Ansible access

UAT:

ansible all -i inventories/uat_hosts.ini -m ping --ask-vault-pass

Production:

ansible all -i inventories/prod_hosts.ini -m ping --ask-vault-pass

Every host should return:

SUCCESS => {
    "changed": false,
    "ping": "pong"
}

Do not continue until all five hosts return pong.

## 5. Verify disks and networking before formatting anything

Select UAT or Production and run read-only checks:

# UAT
export INVENTORY="$PWD/inventories/uat_hosts.ini"

# Production (use this instead for Production)
export INVENTORY="$PWD/inventories/prod_hosts.ini"

ansible-inventory -i "$INVENTORY" --graph

ansible all -i "$INVENTORY" -b -m command \
  -a "lsblk -e7 -o NAME,SIZE,TYPE,FSTYPE,MOUNTPOINTS"

ansible all -i "$INVENTORY" -b -m command \
  -a "ip -brief link"

Confirm:

| Inventory group | UAT hosts | Production hosts | Required data disks |
| --- | --- | --- | --- |
| db_primary, db_standby | U05, U06 | P01, P02 | /dev/sdb, at least 660 GiB usable |
| db_monitor | U07 | P03 | archive disk plus database LVM disk according to environment device variables; deployed Production archive is /dev/sdc1 |
| routing_nodes | U03, U04 | routing P01, P02 | /dev/sdb, at least 120 GiB |

Also confirm:

- /dev/sda is the OS disk;

- the target data disks contain no data to retain;

- the routing interface name matches keepalived_interface;

- UDP 123 reaches the configured NTP servers;

- TCP 443 reaches Ubuntu and PostgreSQL package repositories;

- routing peers can exchange VRRP protocol 112;

- the configured VIP is unused.

Production storage is pre-provisioned and must be inspected rather than reformatted. In the deployed Production monitor, the 405 GB archive filesystem is /dev/sdc1 mounted at /pgdata/WalArchive.

## 6. Step 2 — Deploy UAT

### 6.1 Run static checks

ansible-playbook -i inventories/uat_hosts.ini site.yml \
  --syntax-check --ask-vault-pass

ansible-lint site.yml bootstrap.yml

Resolve all syntax, undefined-variable, and inventory errors before proceeding.

### 6.2 Run the full deployment

ansible-playbook -i inventories/uat_hosts.ini site.yml --ask-vault-pass

Enter the Vault password when prompted.

### 6.3 Expected milestones

Typical elapsed time is 20–60 minutes on newly provisioned VMs. Repository speed, disk speed, DNS, and PostgreSQL initialization can change this estimate.

| Approximate phase | What the output should show |
| --- | --- |
| 1–3 minutes | Input assertions, UFW reset/rule creation, Chrony installation |
| Up to 3 minutes | Each node reaches Chrony Leap status: Normal |
| 3–10 minutes | GPT partitions, LVM, XFS filesystems, persistent mounts |
| 5–20 minutes | PGDG packages and PostgreSQL 18 installed on database hosts |
| 1–5 minutes | Selected-environment monitor created |
| 2–10 minutes | Inventory db_primary host registered |
| 2–20 minutes | Inventory db_standby host clones and joins |
| 2–10 minutes | WAL SSH keys, users, archive configuration |
| 2–10 minutes | PgBouncer, HAProxy, Keepalived, VIP, exporters |

The final recap should show failed=0 and unreachable=0 for every host. changed does not need to be zero on the first run.

Save the deployment transcript:

ansible-playbook -i inventories/uat_hosts.ini site.yml --ask-vault-pass \
  | tee "uat-deployment-$(date +%F-%H%M).log"

## 7. Step 3 — Deploy Production

Production must use reviewed Production IPs, CIDRs, VIP, NTP sources, and Vault secrets. Confirm keepalived_interface, verify that every inventory name has an exact host_ips key, and re-run disk/interface checks first. When Production LVM, XFS, mounts, and /etc/fstab are already provisioned, keep storage_lvm_enabled: false and explicitly skip storage:

ansible-playbook -i inventories/prod_hosts.ini site.yml \
  --syntax-check --ask-vault-pass

ansible-playbook -i inventories/prod_hosts.ini site.yml \
  --skip-tags storage -e storage_lvm_enabled=false --ask-vault-pass \
  | tee "prod-deployment-$(date +%F-%H%M).log"

Do not use --limit for the first deployment. The monitor, primary, and standby plays depend on their declared sequence. The preflight must resolve the five Production hosts listed in Section 1 and must reject every remaining CHANGE_ME guard value.

## 8. Step 4 — Post-deployment verification

Select the environment once before using the commands in this section:

# UAT
export INVENTORY="$PWD/inventories/uat_hosts.ini"
export DB_MONITOR=BHC-QMSSQLU07
export ROUTER_PRIMARY=BHC-PGBSQLU03

# Production (use these three lines instead for Production)
export INVENTORY="$PWD/inventories/prod_hosts.ini"
export DB_MONITOR=BHC-QMSSQLP03.bayshore.ca
export ROUTER_PRIMARY=BHC-PGBSQLP01.bayshore.ca

Confirm the selected inventory before running a test:

ansible-inventory -i "$INVENTORY" --graph

### 8.1 Check for failed services

ansible all -i "$INVENTORY" -b -m command \
  -a "systemctl --failed --no-pager"

No required service should appear in the failed state.

### 8.2 Check PostgreSQL HA state

Query the selected environment’s monitor:

ansible db_monitor -i "$INVENTORY" \
  -b --become-user postgres -m command \
  -a "pg_autoctl show state --pgdata /pgdata/pgroot/data"

Expected:

- both selected-environment data nodes are listed;

- one node has primary state;

- the other has secondary state;

- assigned and reported states settle to stable values.

Check the local systemd service on all database hosts:

ansible db_cluster -i "$INVENTORY" -b -m command \
  -a "systemctl status pg_autoctl --no-pager"

Check synchronous replication from the current primary:

ansible db_primary -i "$INVENTORY" \
  -b --become-user postgres -m shell \
  -a "psql -d postgres -Atc 'SHOW synchronous_commit; SHOW synchronous_standby_names;'"

### 8.3 Check Chrony synchronization and drift

ansible all -i "$INVENTORY" -m command -a "chronyc tracking"
ansible all -i "$INVENTORY" -m command -a "chronyc sources"

Expected:

- Leap status is Normal;

- System time offset is small and stable;

- the selected source in chronyc sources has a ^* marker.

### 8.4 Check LVM, XFS, mounts, and fstab

ansible db_cluster -i "$INVENTORY" -b -m command -a "pvs"
ansible db_cluster -i "$INVENTORY" -b -m command -a "vgs"
ansible db_cluster -i "$INVENTORY" -b -m command -a "lvs"

ansible all -i "$INVENTORY" -b -m shell \
  -a "df -Th /pgdata /pgdata/* 2>/dev/null || true"

ansible db_primary:db_standby -i "$INVENTORY" -b -m command \
  -a "findmnt /pgdata/pgroot/data/pg_wal"

ansible db_cluster -i "$INVENTORY" -b -m command \
  -a "mountpoint /pgdata/pgroot/data"

ansible db_cluster -i "$INVENTORY" -b -m shell \
  -a "stat -c '%d %n' /pgdata/pgroot/data /pgdata/pgroot/backup"

Expected:

- data filesystems are XFS;

- database logical volumes belong to vg_pgdata;

- all required /pgdata/* mount points are present;

- /pgdata/pgroot is the XFS lv_data mountpoint;

- /pgdata/pgroot/data is a normal directory, not a mountpoint;

- /pgdata/pgroot/data and /pgdata/pgroot/backup have the same device ID;

- /pgdata/pgroot/data/pg_wal is a bind mount backed by /pgdata/wal;

- mounts appear in /etc/fstab.

### 8.5 Check UFW policy and role ports

ansible all -i "$INVENTORY" -m command \
  -a "sudo ufw status verbose"

Every host must show:

- Status: active;

- logging enabled;

- default incoming deny;

- default outgoing allow;

- TCP 22 and 9100;

- DB hosts: TCP 5432 and 9187;

- routing hosts: TCP 5432, 6432, and 9127;

- routing hosts: a VRRP rule restricted to the other router.

### 8.6 Check Keepalived VIP ownership

ansible routing_nodes -i "$INVENTORY" -b -m shell \
  -a "ip -brief address show dev {{ keepalived_interface }}"

Use the selected environment’s keepalived_interface. The VIP should appear on exactly one routing node.

Check services:

ansible routing_nodes -i "$INVENTORY" -b -m command \
  -a "systemctl status pgbouncer haproxy keepalived --no-pager"

### 8.7 Check HAProxy

Read the HAProxy runtime socket:

ansible routing_nodes -i "$INVENTORY" -b -m shell \
  -a 'echo "show stat" | socat stdio /run/haproxy/admin.sock'

One PgBouncer backend should be usable for writes; the backend paired with the standby should be marked down by the external primary check.

Check the local statistics page:

ansible routing_nodes -i "$INVENTORY" -m uri \
  -a "url=http://127.0.0.1:8404/stats status_code=200"

Port 8404 is intentionally not opened by UFW for remote access.

### 8.8 Check PgBouncer directly

SSH to a routing node:

ssh "ansible@${ROUTER_PRIMARY}"

Then run:

read -rsp "PgBouncer exporter password: " PGPASSWORD
echo
export PGPASSWORD
psql -h 127.0.0.1 -p 6432 -U pgbouncer_exporter pgbouncer \
  -c "SHOW POOLS;"
unset PGPASSWORD

Repeat on the other routing host. SHOW POOLS should return pool statistics without an authentication error.

### 8.9 Test the complete VIP connection path

Run this from a host inside one of the application_client_cidrs:

read -rsp "Application database password: " PGPASSWORD
echo
export PGPASSWORD
psql "host=<POSTGRESQL_VIP> port=5432 dbname=qms user=qms_app sslmode=require" \
  -c "SELECT inet_server_addr(), inet_server_port(), pg_is_in_recovery();"
unset PGPASSWORD

Replace <POSTGRESQL_VIP> with the environment VIP. Expected:

- connection succeeds through VIP → HAProxy → PgBouncer → PostgreSQL;

- pg_is_in_recovery is false;

- inet_server_addr identifies the current primary.

### 8.10 Check Prometheus exporter endpoints

From a host inside prometheus_scrape_cidr:

curl -fsS http://<ANY_NODE_IP>:9100/metrics | head
curl -fsS http://<DB_NODE_IP>:9187/metrics | head
curl -fsS http://<ROUTING_NODE_IP>:9127/metrics | head

### 8.11 Check WAL archiving

On the current primary:

sudo -u postgres psql -d postgres -c "SELECT pg_switch_wal();"
sudo -u postgres psql -d postgres -c \
  "SELECT archived_count, failed_count, last_archived_wal, last_failed_wal FROM pg_stat_archiver;"

On the selected environment monitor:

sudo -u postgres ls -lh /pgdata/WalArchive | tail

failed_count should not increase and a new WAL segment should appear.

### 8.12 Run the automated read-only health test

The test wrappers default to UAT. Always set INVENTORY explicitly for Production and inspect the graph before execution:

ansible-inventory -i "$INVENTORY" --graph
bash tests/run_health.sh --ask-vault-pass

For a one-command Production invocation:

INVENTORY="$PWD/inventories/prod_hosts.ini" \
  bash tests/run_health.sh --ask-vault-pass

Do not run run_routing_failover.sh, run_db_failover.sh, or RUN_DISRUPTIVE=true tests/run_all.sh in Production without an approved maintenance window.

## 9. Optional controlled failover test

Perform this only in an approved maintenance window.

- Record the current primary:

- ansible "$DB_MONITOR" -i "$INVENTORY" -b --become-user postgres \
  -m command -a "pg_autoctl show state --pgdata /pgdata/pgroot/data"

- On the current primary, stop pg_autoctl:

- sudo systemctl stop pg_autoctl

- Watch state on the selected environment monitor:

- watch -n 2 \
  "ansible '$DB_MONITOR' -i '$INVENTORY' -b --become-user postgres \
  -m command -a 'pg_autoctl show state --pgdata /pgdata/pgroot/data'"

- Repeat the VIP SQL test. It must reach the promoted node and return pg_is_in_recovery() = false.

- Restore the old primary:

- sudo systemctl start pg_autoctl

- Wait for it to rejoin as a stable secondary.

Do not use pg_ctl promote; pg_auto_failover owns state transitions.

## 10. Troubleshooting and common gotchas

### SSH host-key prompt or changed host key

Ansible sets host_key_checking = False, so bootstrap should not block on an interactive prompt. Direct ssh commands may still prompt.

For a brand-new host, verify the fingerprint through the approved VMware or server-build record, then accept it.

If a VM was rebuilt and the old key is cached:

ssh-keygen -R <ENVIRONMENT_HOSTNAME>
ssh-keygen -R <HOST_IP>
ssh-keyscan -H <HOST_IP> >> ~/.ssh/known_hosts

Verify the new fingerprint through a trusted channel before accepting it.

### Bootstrap reports authentication failure

Check:

ssh ansible@<HOST_IP>
sudo -n true
ls -l /etc/sudoers.d/ansible

The same ansible password must work on all targeted hosts when one --ask-pass prompt is used.

### Chrony synchronization times out

Check:

sudo systemctl status chrony --no-pager
chronyc tracking
chronyc sources -v
getent hosts 0.ca.pool.ntp.org

Confirm DNS and outbound UDP 123. If public pools are blocked, replace ntp_servers with approved internal NTP names and rerun site.yml.

### Standby waits for primary registration and times out

Do not delete PGDATA immediately. Check the authoritative monitor state:

ansible "$DB_MONITOR" -i "$INVENTORY" -b --become-user postgres \
  -m command -a "pg_autoctl show state --pgdata /pgdata/pgroot/data"
ansible "$DB_MONITOR" -i "$INVENTORY" -b -m command \
  -a "journalctl -u pg_autoctl -n 200 --no-pager"

On the inventory db_primary host:

sudo systemctl status pg_autoctl --no-pager
sudo journalctl -u pg_autoctl -n 200 --no-pager
sudo -u postgres pg_isready -h 127.0.0.1 -p 5432

Verify:

- the inventory db_primary host is registered on the monitor;

- both data nodes can reach the selected monitor on TCP 5432;

- monitor password and URI values match;

- clocks are synchronized;

- UFW permits cluster_cidr;

- PostgreSQL HBA contains the Ansible-managed pg_auto_failover block.

After fixing the cause, rerun the complete play:

ansible-playbook -i "$INVENTORY" site.yml --ask-vault-pass

### Both HAProxy backends are down

On a routing node:

sudo -u haproxy env HAPROXY_SERVER_ADDR=<ROUTER_IP> \
  /usr/local/sbin/check-pg-primary
echo $?

Exit status 0 means the router’s paired database is primary. Check /etc/haproxy/pg-primary-check.env, direct PostgreSQL reachability, the haproxy_check login, and pg_is_in_recovery().

### UFW removed an emergency rule

This is expected when ufw_reset_rules: true. Add the approved exception to the ufw_firewall role or its variables, review it, and rerun the playbook.

### WAL archiving fails

From the current primary:

sudo -u postgres ssh \
  -i /var/lib/postgresql/.ssh/id_ed25519_wal_archive \
  postgres@<MONITOR_IP> "test -w /pgdata/WalArchive"

sudo -u postgres psql -d postgres -c "SELECT * FROM pg_stat_archiver;"

Check SSH host keys, key authorization, selected-monitor archive ownership, disk space, and network access.

## 11. Safe re-runs

The playbook is designed to be re-run after correcting a failure:

ansible-playbook -i "$INVENTORY" site.yml --ask-vault-pass

Important behavior:

- storage modules preserve existing matching partitions, LVs, filesystems, and mounts;

- creates guards prevent normal PostgreSQL reinitialization;

- templates restart services only when configuration changes;

- polling waits for monitor and node readiness;

- UFW is intentionally reset and rebuilt on every full run;

- the database/WAL play uses serial: 1 to avoid restarting database hosts simultaneously.

Do not manually remove LVM objects, PGDATA, pg_autoctl state, or archive keys unless an approved rebuild procedure explicitly requires it.